Electricity Market Design: How do we get electricity markets to respect the levels of reliability that customers are willing to pay for?

Julia Frayer, Managing Director at London Economics International LLC, +1.617413.8628 julia@londoneconomics.com

Abstract

Electricity markets are increasingly misaligned with the technological and economic realities of modern power systems. Despite rapid digitalization, distributed energy resources, and advanced demand-side capabilities, wholesale electricity market designs continue to rely on stylized textbook constructs and very old fashioned about final consumers’ ability to express their preferences as to the reliability they are willing to pay for   — which is most notable in the continued use a single Value of Lost Load (VoLL) parameter applied uniformly across heterogeneous consumers. This paper asks: how can electricity markets be redesigned to better reflect and operationalize consumers’ differentiated willingness to pay for reliability?

Methods

We will begin this paper by explaining how VoLL estimates are used in modern competitive electricity markets, based on economic literature survey and review of market rules around the world. We will then develop an evidence-based analysis of why a single VoLL measure is inadequate at ensuring social welfare maximizing outcomes.  The analytical framework will rely on a combination of conceptual modeling as well as simulation-based analysis of actual power systems, with real world facets and attributes. We intend to test alternatives to the current practices by leveraging the theoretical requirements of a two-sided market in which consumers (or their aggregators) can express time- and state-contingent willingness to pay for electric reliability, and flexible loads can offer reliability services in return within the constraints of known programmatic market features (like aggregated demand response and virtual power plant aggregation) and limits of currently available technology (distributed energy resources, smart meters). We will show how these market features rather than relying on a single administratively determined VoLL, enables voluntary expression of reliability within the energy market.  Using a simulation-based approach, we compare outcomes under a uniform VoLL regime to those under a differentiated system of customer-specific expression of the reliability levels they want.

Findings and Recommendations

Although the research is ongoing, the author observes preliminary modeling indicates that allowing differentiated reliability preferences to be expressed within the market increases allocative efficiency, reduces welfare losses associated with involuntary curtailment, and enhances investment signals for flexible demand and distributed resources. Importantly, small consumers need not engage in real-time optimization; automated aggregation and default service options can preserve simplicity while enabling participation. The findings suggest that moving toward consumer-centric, two-sided market design can improve both efficiency and equity in electricity systems undergoing deep decarbonization and digitalization.

References

Agency for the Cooperation of Energy Regulators (ACER). (2020). Methodology for calculating the value of lost load (VoLL), the cost of new entry (CONE) and the reliability standard (RS).

Argonne National Laboratory. (2024). Planning and Operations in Electricity Markets Under System Transformation.

Armstrong, M. (2006). Competition in two-sided markets. RAND Journal of Economics, 37(3), 668–691.

Australian Energy Market Commission. (2019). What next for a two-sided energy market? The implications of venturing behind the meter.

Autoridad Nacional de los Servicios Públicos (ASEP), Panamá. (2018). Estudio del costo de la energía no suministrada (CENS)—Informe final.

Brattle Group. (2024). Value of Lost Load Study for the ERCOT Region.

Burns, R. M., & Gross, G. (1990). Value of service reliability. IEEE Transactions on Power Systems.

Carvallo, JP. Lawrence Berkeley National Laboratory. (2024). The Value of Lost Load - Concepts, methods, and applications.

Comisión Nacional de Energía (CNE), Chile. (2025). Estudio: Costo de falla de corta y larga duración en el SEN y SSMM—Informe final.

Gorman, W. (2022). The quest to quantify the value of lost load: A critical review of the economics of power outages. The Electricity Journal, 35(8), 107187.

Joskow, P. L., & Tirole, J. (2007). Reliability and competitive electricity markets. RAND Journal of Economics, 38(1), 60–84.

Khorasany, Mohsen et al. (2020). Paving the Path for Two-Sided Energy Markets: An Overview of Different Approaches. IEEE Access, Volume 8.

Larsen, P. H., LaCommare, K. H., Eto, J., & Sweeney, J. LBNL. (2015). Updated value of service reliability estimates for electric utility customers in the United States.

Leahy, E., & Tol, R. S. J. (2015). Value of lost load: An efficient economic indicator for power interruption costs? Frontiers in Energy Research, 3, 55.

Littlechild, S. (2017). Two-sided markets and the utility of the future: How services and platforms can drive next-generation energy systems. The Electricity Journal, 30(10).

London Economics. (2013). The Value of Lost Load (VoLL) for Electricity in Great Britain.

Midcontinent Independent System Operator (MISO). (2024). Scarcity pricing white paper: Value of lost load and operating reserve demand curve.

Rochet, J.-C., & Tirole, J. (2003). Platform competition in two-sided markets. Journal of the European Economic Association, 1(4), 990–1029.

Weyl, E. G. (2010). A price theory of multi-sided platforms. American Economic Review, 100(4), 1642–1672.

Zhou, Ella, Sika Gadzanku, Cabell Hodge, Mike Campton et al. NREL. (2023). Best Practices in Electricity Load Modeling and Forecasting for Long-Term Power System Planning.